# TMS320C5416 DSK Problems and Fixes

This document describes some of the problems that might be encountered while using the TMS320C5416 DSK and suggests ways in which to remedy them.

| Problem 1. |  |  |
| --- | --- | --- |
| Description | Cause of Problem | Suggested Remedy |
| Project will not build | The settings for the build are not correct. It is not possible to copy a project from one directory to another without rebuilding. | Try building again, carefully following each step in the document “Installing the TMS320C5416 Applications” |

| Problem 2. |  |  |
| --- | --- | --- |
| Description | Cause of Problem | Suggested Remedy |
| Audio output is mono where stereo is expected | A block of code is taking too long to execute, either through too much processing to do in the allotted time or C code is not time-efficient enough. | The method to measure the execution times of blocks of code is given in Laboratory 9. Reduce the amount of processing or filter length Replace C code with assembly version. |

| Problem 3. |  |  |
| --- | --- | --- |
| Description | Cause of Problem | Suggested Remedy |
| Digital signal processing code has been added but when the program is run, the processing has no effect | Code has been updated but the old output file .out is being used | Rebuild the project |

| Problem 4. |  |  |
| --- | --- | --- |
| Description | Cause of Problem | Suggested Remedy |
| No audio output on one channel, even though the bargraph shows there is an input when the switches are in position 0 (straight through). | Input signal not physically getting to the jack connectors. (The bargraph is of left and right channels combined) | Use stereo jack plugs rather than mono. Instead of processing left and right inputs, use the mono_input in C code instead. |

| Problem 5. |  |  |
| --- | --- | --- |
| Description | Cause of Problem | Suggested Remedy |
| Program does not load automatically after build | Automatic load of program switched off. This option can become unprogrammed if the DSK fails to start up. | Go to Option -> Customise -> Program Load Options and tick the box to automatically load the program after build. |

| Problem 6. |  |  |
| --- | --- | --- |
| Description | Cause of Problem | Suggested Remedy |
| A particular IIR filter is not working at the expected frequency. | The wrong coefficients have been chosen. | Check that the format in C code implementing the IIR filter is: &IIR_high_pass_1000Hz[0] (Must be preceded by a & and the number in [] must be zero). |

| Problem 7. |  |  |
| --- | --- | --- |
| Description | Cause of Problem | Suggested Remedy |
| The IIR Form II filter has the wrong cut-off frequency or overloading, particularly at low frequencies. | Internal overloading of IIR algorithm | Increase the number of shifts after the first multiply in the IIR code. |

| Problem 8. |  |  |
| --- | --- | --- |
| Description | Cause of Problem | Suggested Remedy |
| IIR filter causing all LEDs of the DSK to be on and noise generated at the audio output. | One of the IIR coefficients has the wrong sign. | Check the IIR coefficients against their theoretical values. |

| Problem 9. |  |  |
| --- | --- | --- |
| Description | Cause of Problem | Suggested Remedy |
| Warning: Last line of file ends without new line | A binary file was used as the basis of the code | Open file with Windows Notepad, modify the file by typing in something and deleting it again. Save the file. |

| Problem 10. |  |  |
| --- | --- | --- |
| Description | Cause of Problem | Suggested Remedy |
| IIR notch filter is unstable | Cannot use value of radius r = 1.000 | Reduce r to less than 1.000 e.g. 0.9500 |

| Problem 11. |  |  |
| --- | --- | --- |
| Description | ## Cause of Problem | Suggested Remedies |
| When assembly language function is called, the program hangs up or parameter not being passed to the function | Wrong value of *SP( ) being used for local variable on the stack. | Make the variable a local variable rather than on the stack. *SP() has been altered by adding PSHM and POPM instructions. The offset needs to be corrected. |

| Problem 12. |  |  |
| --- | --- | --- |
| Description | Cause of Problem | ## Suggested Remedy |
| A function written in C should be returning a long int but instead always returns zero. Especially applies to FIR and adaptive filters. | Missing function prototype. The default return value for a function is int. | Add appropriate .h file to project for the function being used. |

| Problem 13. |  |  |
| --- | --- | --- |
| Description | Cause of Problem | ## Suggested Remedy |
| The DSK has locked up. | The power supply is very sensitive to mains disturbances, for example, when another electrical device is plugged into the adjacent socket. | Plug in and turn on test equipment and powered computer speakers before running the TMS320C5416 DSK |

| Problem 14. |  |  |
| --- | --- | --- |
| Description | Cause of Problem | ## Suggested Remedy |
| When using an example from DSK examples, there is only output on one channel. | The default is for one channel on only. | Ensure that the setup values for the DSK are 0x1FF, not 0x000. DSK5416_PCM3002_Config setup = { 0x1FF, 0x1FF, 0x0, 0x0 }; |