# Answers to Tutorial 12

Tutorial 12.	Stack Operations

| 1. | The stack is an area of data memory used for temporary storage of variables and in particular for subroutine calls. |
| --- | --- |
| 2. | The term push means to temporarily store a variable from either a data memory address or a memory-mapped register on the stack. The stack pointer is decremented as part of the process. |
| 3. | The term pop means to restore a value from the top of the stack to either a data memory address or a memory-mapped register. The stack pointer is incremented as part of the process. |
| 4. | For correct operation, the operand must be a data memory address. a) PSHD 2h ; Correct. b) PSHD AR0 ; Cannot use a memory-mapped register. c) PSHD AR0+ ; Incorrect. Missing *. d) PSHD *AR1- ; Correct. e) PSHD *AR2+ ; Correct. f) PSHD SP ; Cannot use a memory-mapped register. |
| 5. | For correct operation, the operand must be a data memory address. a) POPD 34h ; Correct. b) POPD AR1 ; Cannot use a memory-mapped register. c) POPD AR4+ ; Incorrect. Missing *. d) POPD AR6- ; Incorrect. Missing *. e) POPD *AR2 ; Correct. f) POPD *AR3+ ; Correct. g) POPD *AR7 ; Correct. |
| 6. | For correct operation, the operand must be a memory-mapped register. a) PSHM *AR1 ; Cannot use data memory address. b) PSHM AR4 ; Correct. c) PSHM AR3+ ; Incorrect. Cannot use + operator. d) PSHM *AR2- ; Cannot use data memory address. e) PSHM SP ; Correct. f) PSHM ST0 ; Correct. |
| 7. | For correct operation, the operand must be a memory-mapped register. a) POPM 22h ; Cannot use a data memory address. b) POPM AR4 ; Correct. c) POPM AR0- ; Incorrect. Cannot use - operator. d) POPM *AR1+ ; Cannot use a data memory address. e) POPM SP ; Correct. f) POPM ST0 ; Correct. g) POPM ST1 ; Correct. |
| 8. | The top of stack refers to the last value pushed onto the stack. |
| 9. | When push and pop instructions are used in pairs, the stack pointer is restored to its original value. |
| 10. | To swap the values in data memory addresses 100h and 101h without using an accumulator we can write: LD #2, DP ; Page 2. Gain access to data memory ; addresses 100h to 101h. PSHD 0h ; Push first value. PSHD 1h ; Push second value. POPD 0h ; Pop first value. POPD 1h ; Pop second value. |
| 11. | To copy the contents of a memory-mapped register to a data memory address we can use the instructions PSHM and POPD e.g.: PSHM AR0 ; Push AR0 onto the stack. POPD 2h ; Pop top of stack to data memory address. |
| 12. | The TMS320C5000 uses the instruction PSHM to push the contents of an accumulator onto the stack. This is because both parts of each accumulator are memory mapped registers. PSHM AL ; Push low word of accumulator A. PSHM AH ; Push high word of accumulator A. PSHM BL ; Push low word of accumulator B. PSHM BH ; Push high word of accumulator B. |