# Answers to Tutorial 3

Tutorial 3. 	Logical Operations

| 1. | The logical AND is used to clear to zero any bits in which we are not interested. This operation is often used before a test . |
| --- | --- |
| 2. | The result of performing a logical AND of FFFF3333h with FFh is 00000033h. |
| 3. | The result of performing a logical OR of 55555555h with FFFFh is 5555FFFFh. |
| 4. | The result of performing a logical XOR of 77770000h with 1111h is 77771111h. |
| 5. | The result of performing a logical XOR of 000055AAh with FFFFh is 0000AA55h. |
| 6. | To use the logical XOR as an inverter, set the particular bits to be inverted to 1. |
| 7. | The logical XOR of two values performs a bitwise addition of them. |
| 8. | The logical AND affects both the high and low words of the accumulator while the instructions AND and XOR do not. |
| 9. | The instruction XOR only affects the low word of an accumulator and therefore cannot be used to invert the full 32 bits. |
| 10. | The term complement means that all bits that are 0 are changed to 1 and all bits that arer 1 are changed to 0. |
| 11. | If accumulator A contains the value 00007777h, then after the instruction CMPL it then contains FFFF8888h. |
| 12. | To perform a logical AND of data memory address 61h with the constant 3FFFh we use the instruction ANDM #3FFFh, 61h. |
| 13. | To perform a logical OR of data memory address 72h with the constant FF00h using a single instruction we use the instruction ORM #0FF00h, 72h. |
| 14. | To invert the contents of data memory address 64h we use the instruction XORM #0FFFFh, 64h. |