# Answers to Tutorial 4

Tutorial 4 	Signed Numbers and Shifting Data

| 1. | An unsigned number can be zero or positive only. A signed number can take negative, zero or positive values. |
| --- | --- |
| 2. | The instruction SSBX SXM turns on sign-extension mode. |
| 3. | The instruction RSBX SXM turns off sign-extension mode. |
| 4. | The instruction SFTL means shift accumulator logically. |
| 5. | The instruction SFTL 1, A shifts the accumulator one place to the left. |
| 6. | The instruction SFTL -1, A shifts the accumulator one place to the right. |
| 7. | To shift the contents of accumulator B two places to the right we write: SFTL -2, B. |
| 8. | To shift the high word of accumulator A into the low word of accumulator A we use the instruction SFTL -16, A. |
| 9. | Sign-extension mode does not affect the instruction SFTL (shift accumulator logically). |
| 10. | When sign-extension mode is turned on, when the instruction SFTA is executed, the value of the sign bit (the most significant or left-most bit) is maintained. When signed extension mode is turned off a simple shift is produced. |
| 11. | The logical shift SFTL is used for general purpose shift while the arithmetic shift SFLA must be used with signed numbers. |
| 12. | When the instruction LD is used with a shift, the constant is shifted then loaded into the accumulator. |
| 13. | The default shift for the instruction LD is zero. Therefore the instructions LD #5555h, A and LD #5555h, 0, A are equivalent. |
| 14. | To shift the value 7777h into the high word of accumulator B we use the instruction LD #7777h, 16, B. |
| 15. | To load the low word of an accumulator with a constant there are two instructions we can use: LD #5555h, A or XOR #5555h, A. |
| 16. | To load accumulator B with the constant FEDC9876h we write: LD #0FEDCh, 16, B ; Load high word of accumulator B. OR #9876h, B ; Load low word only. |