# Answers to Tutorial 5

Tutorial 5	Arithmetic Operations

| 1. | To add the constant 7Fh to accumulator A we write ADD #7Fh, A |
| --- | --- |
| 2. | With sign-extension mode turned off, if the constant FFFFh is added to 1h in accumulator A, the result will be 10000h. |
| 3. | The instruction SUB #100h, A subtracts the constant 100h from accumulator A. |
| 4. | When sign-extension mode is turned off, if the constant 1 is subtracted from accumulator B, accumulator B then contains FFFEh. |
| 5. | The instruction ADDM #10, *AR4 adds the immediate value 10 to the contents of the data memory address pointed to by auxiliary register AR4. |
| 6. | The letter M in the instruction STM means store in memory-mapped register. The letter M in the instruction ADDM means (data) memory. |
| 7. | To add the constant 1 to the contents of data memory address 300h without using an accumulator we write: LD #6, DP then ADDM #1, 0h. |
| 8. | To subtract the constant 1 from the contents of a data memory address we add the value -1 (FFFFh) e.g. ADDM #0FFFFh, *AR2. However, we must first turn on sign-extension mode. |
| 9. | The instruction MAR means Modify Auxiliary Register. |
| 10. | The symbol * is used to indicate that the auxiliary register is used as part of the operation: a) With the instruction LD, the symbol * is used to indicate indirect addressing. b) With the instruction MAR, the symbol * is used to indicate that the register itself is being modified. |
| 11. | The instruction MAR is used to alter the contents of an auxiliary register by incrementing or decrementing it. The instruction STM is used to store a value in an auxiliary register. |
| 12. | The instruction CMPL is a logical operation used to convert all the bits that are 0 to 1 and vice versa. The instruction NEG is used with signed arithmetic to convert a positive number to a negative number. |