Tutorial 5: Arithmetic Operations

New Instructions Introduced

ADD		ADDM

SUB

MAR

NEG

New Flags Introduced

OVM

Overview of Tutorial

In earlier tutorials we saw how to load data and carry out logical operations. We shall now see how to carry out additions and subtractions on the contents of accumulators, data memory addresses and auxiliary registers.

Simple Addition

To add an immediate value to accumulator A or accumulator B, we use the instruction ADD (add to accumulator).

Example 5-1.

| LD #1000h, A | ; Load accumulator A with 00001000h. |
| --- | --- |
| ADD #3Fh, A | ; Add the value 3Fh to accumulator A. ; Accumulator A now contains 103Fh. |

The instruction ADD takes two operands. The first operand is a constant  and the second operand is the accumulator to be used.

The instruction ADD is affected by sign-extension mode. The constant is taken to be unsigned when sign-extension mode is off (SXM = 0)and signed when sign-extension mode is on (SXM = 1). This means that the constant can be either be unsigned (in the range 0 to 65535 decimal) or signed (in the range -32768 to +32767 decimal). For the time being, we shall only consider unsigned values. When the constant lies in the range 0 to +32767 (0 to 7FFFh) then sign-extension mode makes no difference.

We can also add the contents of a data memory address to an accumulator using the instruction ADD (add to accumulator) with direct addressing. Say that data memory address 61h already contains the value 3Fh, then to carry out an addition we write:

Example 5-2.

| LD #0, DP | ; Page 0. Gain access to data memory ; addresses 60h to 7Fh. |
| --- | --- |
| LD #1000h, B | ; Load accumulator B with 00001000h. |
| ADD 61h, B | ; Add the value stored at data memory ; address 61h to accumulator B. ; Accumulator B now contains 1000h + ; 3Fh = 103Fh. |

The instruction ADD (add to accumulator) also supports indirect addressing. We can therefore carry out the same operation as carried out in Example 5-2, but this time using indirect addressing. Assuming that data memory address 61h contains the value 3Fh:

Example 5-3.

| STM #61h, AR3 | ; Load auxiliary register AR3 with the ; address of data memory location ; 61h. AR3 = 61h. |
| --- | --- |
| LD #1000h, A | ; Load accumulator A with 00001000h. |
| ADD *AR3, A | ; Add the value contained in the data ; memory address pointed to by AR3 to ; accumulator A. Accumulator A now ; contains 1000h + 3Fh = 103Fh. |

When adding values greater than +32767 (7FFFh), sign-extension mode must be turned off, otherwise the constant will be treated as a negative number.

Example 5-4.

| RSBX SXM | ; Turn off sign-extension mode. |
| --- | --- |
| LD #1000h, A | ; Load accumulator A with 00001000h. |
| ADD #0FFFFh, A | ; Add 65535 (FFFFh) to accumulator A. ; Accumulator A now contains 00010FFFh. |

Signed Additions

When sign-extension mode is turned on, the immediate value used with the instruction ADD is treated as a signed number. Any values between 8000h and FFFFh are taken to be negative numbers; 8000h represents -32768 and FFFFh represents -1.

Whereas Example 5-4 has been used to add the positive number 65535 to accumulator A, Example 5-5 shows how to add the negative number -1 (FFFFh) to accumulator A.

Example 5-5.

| SSBX SXM | ; Turn on sign-extension mode. |
| --- | --- |
| LD #1000h, A | ; Accumulator A now contains 00001000h. |
| ADD #0FFFFh, A | ; Add -1 to accumulator A. Accumulator A ; now contains 00000FFFh. |

When sign-extension mode is turned on, the largest positive value we can add to an accumulator is +32767 (7FFFh).

Example 5-6.

| SSBX SXM | ; Turn on sign-extension mode. |
| --- | --- |
| LD #7FFFh, B | ; Load accumulator B with maximum ; positive value (+32767). |
| ADD #7FFFh, B | ; Add +32767 (7FFFh) to accumulator B. ; Accumulator B now contains 0000FFFEh. |

Simple Subtraction

In a similar way to which we carried out addition, we can subtract an immediate value from either accumulator A or accumulator B. For this we can use the instruction SUB (subtract from accumulator). Again, the instruction SUB is affected by sign-extension mode. If the value being subtracted lies in the range 0 to +32767, then sign-extension mode has no effect.

Example 5-7.

| LD #7EDCh, A | ; Load accumulator A with 00007EDCh. |
| --- | --- |
| SUB #56h, A | ; Subtract 56h from accumulator A. ; Accumulator A now contains 00007E86h. |

When sign-extension mode is turned off, the constant lies in the range 0 to 65535 (0 to FFFFh). When sign-extension mode is turned on, the constant to be subtracted must lie in the range -32768 to +32767 (8000h to 7FFFh). This means that the largest positive value that can be subtracted is 7FFFh.

We can subtract the contents of a data memory address from an accumulator using the instruction SUB (subtract from accumulator). For example, to subtract the contents of data memory address 71h from accumulator B we can write:

Example 5-8.

| RSBX SXM | ; Turn off sign-extension mode. |
| --- | --- |
| LD #1, DP | ; Page 1. Access to data memory ; addresses 80h to FFh. |
| ST #8000h, 71h | ; Store the value 8000h (a positive ; number) at data memory address 80h + ; 71h = F1h. |
| LD #0FF00h, B | ; Load accumulator B with 0000FF00h (a ; positive number). |
| SUB 71h, B | ; Subtract the unsigned contents of data ; memory address 80h + 71h = F1h from ; accumulator B. The result 00007F00h is ; now in accumulator B. |

We can perform a similar operation as shown in Example 5-8, but instead using indirect addressing and with a signed value in data memory:

Example 5-9.

| SSBX SXM | ; Turn on sign-extension mode. |
| --- | --- |
| STM #0F1h, AR4 | ; Store the address of data memory ; location F1h in AR4. AR4 = F1h. |
| ST #3000h, *AR4 | ; Store 3000h (a positive number) at the ; data memory address pointed to by ; auxiliary register AR4. |
| LD #8F00h, B | ; Load accumulator B with FFFF8F00h (a ; negative number). |
| SUB *AR4, B | ; Subtract the signed contents of the ; data memory address pointed to by AR4 ; from accumulator B. The result ; FFFF5F00 is now in accumulator B. |

Adding Directly to Memory

We may wish to perform an addition on the contents of a memory address. An example of such an operation is given using C code in Example 5-10:

Example 5-10.

| unsigned int value; |  |
| --- | --- |
| value += 0x00FF; | // Add constant FFh to value. |

The instruction ADD can only be applied to either accumulator A or to accumulator B. To perform add an immediate value to a data memory address, we must use the instruction ADDM (add long immediate value to memory). Example 5-10 has been implemented in assembly language in Example 5-11 and assumes that the variable value is stored at data memory address 222h:

Example 5-11.

| LD #4, DP | ; Page 4. Access to data memory ; addresses 200h to 27F. |
| --- | --- |
| ADDM #0FFh, 22h | ; Add FFh to the contents of data ; memory address 200h + 22h = 222h. The ; result is placed at data memory ; address 222h. |

Here the letter M in the instruction ADDM means memory, not memory-mapped register. The instruction ADDM takes two operands. The first operand is the immediate value and the second operand is the data memory address. The result of the addition is placed in the data memory address.

The instruction ADDM is affected by sign-extension mode. When sign-extension mode is turned off, the constant is taken to be positive in the range 0 to 65535 (0 to FFFFh). When sign-extension mode is on, the constant is taken as being a signed value in the range -32768 to +32767 decimal (8000h to 7FFFh).

We can carry out the same operation in Example 5-11, but this time using indirect addressing:

Example 5-12.

| STM #222h, AR1 | ; Store address of data memory location ; 222h in auxiliary register AR1. |
| --- | --- |
| ADDM #0FFh, *AR1 | ; Add FFh to the contents of data ; memory address pointed to by auxiliary ; register AR1. The result is placed at ; data memory address pointed to by AR1. |

Subtraction from Data Memory

We may also wish to perform a subtraction of a constant from the contents of a memory location. Example 5-13 shows how this would be done in C code:

Example 5-13.

| signed int value; |  |
| --- | --- |
| value -= 0x0001; | // Subtract constant 1 from value. |

The TMS320C5000 does not provide an instruction to subtract an immediate value (constant) directly from the contents of a data memory address. Instead we must use the instruction ADDM (add long immediate value to memory) with a negative constant. An implementation of Example 5-13 using assembly language is shown in Example 5-14. Assuming the variable value  is stored at data memory address 284h:

Example 5-14.

| SSBX SXM | ; Turn on sign-extension mode. |
| --- | --- |
| LD #5, DP | ; Page 5. Access to data memory ; addresses 280h to 2FFh. |
| ADDM #0FFFFh, 4h | ; Add FFFFh (-1) to the contents ; of data memory address 280h + 4h = ; 284h. The result is placed in data ; memory address 284h. |

The same operation as shown in Example 5-14 can be carried out using indirect addressing. This is shown in Example 5-15:

Example 5-15.

| SSBX SXM | ; Turn on sign-extension mode. |
| --- | --- |
| STM #284h, AR2 | ; Store the address of data memory ; location in auxiliary register AR2. ; AR2 = 284h. |
| ADDM #-1, *AR2 | ; Add FFFFh (-1) to the contents ; of data memory address pointed to by ; auxiliary register AR2. The result ; is placed in data memory address ; pointed to by AR2. |

Preventing Overflow During Addition and Subtraction

Consider the following simple problem. Accumulator A contains 7FFFFFFFh and we add 1. The result will be 80000000h. If the number in accumulator A represents a signed number, then adding 1 to 7FFFFFFFh (a large positive number) will produce the result of 8000000h (a large negative number). The addition causes the contents of the accumulator to overflow beyond the upper limit of signed values.

The TMS320C5000 provides a mechanism to prevent overflows. This is done by setting the overflow flag in status register ST1 to 1 to turn on overflow-mode (OVM = 1). In all the examples given in this tutorial so far, it has been assumed that overflow-mode has been switched off (OVM = 0). The instructions ADD, SUB and ADDM are all affected by overflow-mode.

When overflow mode is turned on, the instruction ADD imposes an upper value of 7FFFFFFFh in the accumulators, whether or not sign-extension mode is on.

Assuming that accumulator A already contains 7FFFFFFFh (the maximum signed value that can be held in the 32 bits of an accumulator) then:

Example 5-16.

| SSBX OVM | ; Turn on overflow-mode. |
| --- | --- |
| ADD #1, A | ; Add 1 to accumulator A. Accumulator A ; still contains 7FFFFFFFh. The sum has ; been limited to a maximum value of ; 7FFFFFFFh. |

When an addition occurs that would take the value in an accumulator over its maximum, then the accumulator limits itself to 7FFFFFFFh. This is preferable to going to 80000000h which can represent a negative number.

In a similar way, the instruction SUB imposes a lower limit of 80000000h, whether or not sign-extension mode is on.

Assuming that accumulator A already contains 80000000h (the most negative signed value that can be held in the 32 bits of an accumulator) then:

Example 5-17.

| SSBX OVM | ; Turn on overflow-mode. |
| --- | --- |
| SUB #1, A | ; Subtract 1 from accumulator A. ; Accumulator A still contains ; 80000000h. The subtraction has been ; limited to a minimum value of ; 80000000h. |

When an subtraction occurs that would take the value in an accumulator over its minimum, then the accumulator limits itself to 80000000h. Again this is preferable to the value in the accumulator changing sign and becoming 7FFFFFFFFh which can represent a positive number.

When using the instruction ADDM, overflow mode limits the contents of a data memory address to a maximum of 7FFFh and a minimum of 8000h.

Example 5-18.

| SSBX OVM | ; Turn on overflow mode. |
| --- | --- |
| LD #3, DP | ; Page 3. Gain access to data memory ; addresses 180h to 1FFh. |
| ST #7FFFh, 30h | ; Store the value 7FFFh in data memory ; address 180h + 30h = 1B0h. |
| ADDM #1h, 30h | ; Add 1 to the contents of data ; memory address 1B0h. Data memory ; address 1B0h now contains 7FFFh. |

Incrementing and Decrementing Auxiliary Registers

We have seen how to carry out additions and subtractions using accumulators and values stored in data memory. We shall now carry out additions and subtractions on the auxiliary registers AR0 to AR7.

Should we wish to increment or decrement the contents of an auxiliary register without affecting the contents of the data memory address, we use the instruction MAR (modify auxiliary register). This takes a single operand as shown in Example 5-19:

Example 5-19.

| MAR *AR3+ | ; Increment AR3. |
| --- | --- |
| MAR *AR3- | ; Decrement AR3. |
| MAR *AR3 | ; Neither decrements nor increments AR3. |
| MAR AR3+ | ; Illegal. Missing *. |

The analogy of C code would be:

Example 5-20.

| unsigned int *AR3; | // AR3 is a pointer to memory. |
| --- | --- |
| AR3++; | // Point to next higher address. |
| AR3--; | // Point to next lower address. |

Here the letter M in the instruction MAR means modify, rather than move, as is the case with some other processors. Please note that the description given here of the instruction MAR only covers some of the capabilities of the instruction MAR.  More details as to the usage of the instruction MAR are given in the TMS320C5000 databook.

An important point to remember is that when using the instruction MAR (modify auxiliary register), the symbol * does not refer to indirect addressing. It simply means an operation is carried out on the auxiliary register.

Modifying Auxiliary Registers

In order to add a constant from the value in an auxiliary register, we use a special variation of the instruction MAR that adds the value in AR0 to the auxiliary register.

Example 5-21 shows how to add together the contents of two auxiliary registers.

Example 5-21.

| STM #100, AR4 | ; Store the constant 100 decimal in AR4. |
| --- | --- |
| STM #10, AR0 | ; Store the constant 10 decimal in AR0. |
| MAR *AR4+0 | ; Add AR4 to AR0 and store sum in AR4. ; AR4 now contains 100 + 10 = 110 (6Eh). |

The instruction MAR *AR4+0 modifies auxiliary register AR4 by adding the contents of AR0 to AR4 and then putting the sum in AR4.

Similarly, we can subtract a value from an auxiliary register.

Example 5-22.

| STM #100, AR2 | ; Store the constant 100 decimal in AR2. |
| --- | --- |
| STM #20, AR0 | ; Store the constant 20 decimal in AR0. |
| MAR *AR2-0 | ; Subtract AR0 from AR2 and store result in ; AR2. AR2 now contains 100-20 = 80 (50h). |

The value to be added or subtracted must always be put in AR0. The sum or difference will always be stored in the register listed first in instruction MAR.

Example 5-23.

| MAR *AR7+0 | ; Add AR0 to AR7 and store sum in AR7. |
| --- | --- |
| MAR *AR6-0 | ; Subtract AR0 from AR6 and store result in ; AR6. |
| MAR *AR0+0 | ; Add AR0 to AR0 and store sum in AR0. |
| MAR *AR3+4 | ; Illegal attempt to subtract AR4 from AR3. |

The operator on the auxiliary register of the type *AR5+0 can also be used with some of the other instructions already introduced.

Example 5-24.

| STL A, *AR4+0 | ; Store the low word of accumulator A in ; the data memory address pointed to by ; AR4. Add contents of AR0 to AR4. |
| --- | --- |
| STH A, *AR5-0 | ; Store the high word of accumulator A in ; the data memory address pointed to by ; AR5. Subtract the contents of AR0 from ; AR5. |
| ST #400h, *AR6+0 | ; Store 400h at the data memory address ; pointed to by AR6. Add the contents of ; AR0 to AR6. |
| ST #32h, *AR7-0 | ; Store 32h at the data memory address ; pointed to by AR7. Subtract the contents ; of AR0 from AR7. |
| STM #99h, *AR1-0 | ; Illegal. Not allowed to modify auxiliary ; register with instruction STM. |

In each of the sample instructions in Example 5-24, the auxiliary register is modified after the store operation has been done.

Differences Between  the Instructions STM and MAR

When first learning TMS320C5000 assembly language, the differences between the instructions STM (store in memory-mapped register) and MAR (modify auxiliary register) may not be immediately obvious.

The instruction STM stores a value in a memory-mapped register AR0 to AR7. It is not possible to modify an auxiliary register as part of this instruction.

Example 5-25.

| STM #33h, AR0 | ; Store the immediate value 33h in ; memory-mapped register AR0. |
| --- | --- |
| STM #33h, AR0+ | ; Illegal. Not allowed to modify AR0. |

By contrast, the instruction MAR modifies an auxiliary register but does not store any data.

Example 5-26.

| MAR *AR0- | ; Decrement the contents of auxiliary ; register AR0. No data has been ; stored. |
| --- | --- |

A practical example of how we would use the instructions STM and MAR is to implement a for loop in C using an auxiliary register to store our variable. Consider part of the for loop shown in Example 5-27:

Example 5-27.

|  | unsigned int x; |
| --- | --- |
|  | for ( x = 0 ; x < 10; x++) |
|  | { /* Body of for loop */} |

To partly implement this in assembly language we will use auxiliary register AR4 to store the variable x.

Example 5-28.

| STM #0, AR4 | ; Store the value zero in memory- ; mapped register AR4. This is ; equivalent to x = 0. |
| --- | --- |
| MAR *AR4+ | ; Increment the contents of auxiliary ; register AR4. This is equivalent to ; x++. |

In the case where we use an auxiliary register to store the C variable, no operations are performed on data memory.

Converting Numbers from Unsigned to Signed

Within a program, there is sometimes the need to convert a positive number to a negative number. For example, as part of an algorithm we sometimes have a variable that is positive at times, and negative at others. As a simple problem, how do we convert the positive number +1 to the negative number -1?

Expressed as a 16-bit number, +1 is 0001h and -1 is FFFFh. To convert a positive number to a negative number, one way is to complement the number and then add 1 to it. This is shown in Example 5-29:

Example 5-29.

| LD #1h, A | ; Load accumulator A with 00000001h. |
| --- | --- |
| CMPL A | ; Complement accumulator A. ; Accumulator A now contains ; FFFFFFFEh. |
| ADD #1, A | ; Add 1 to accumulator A. Accumulator ; A now contains FFFFFFFFh (-1). |

The instruction CMPL is not affected by sign-extension mode.

In order to convert an unsigned number to a signed number, the TMS320C5000 provides the instruction NEG (negate accumulator):

Example 5-30.

| LD #1h, A | ; Load accumulator A with 00000001h. |
| --- | --- |
| NEG A | ; Negate accumulator A. ; Accumulator A now contains ; FFFFFFFFh (-1 decimal). |

Again the instruction NEG is not affected by sign-extension mode.

Upgrading from the TMS320C2000 to the TMS320C5000

The TMS320C5000 has two accumulators, whilst earlier devices had only one. This means that earlier logical and arithmetic instructions designed for use with a single accumulator cannot be used on the TMS320C5000. Table 5-1 gives a comparison of instructions:

Table 5-1.  Comparison of Instructions

| Description | TMS320C2000 | TMS320C5000 Instruction |
| --- | --- | --- |
| Add constant to accumulator | ADD #100h | ADD #100h, Acc |
| Subtract constant from accumulator | SUB #100h | SUB #100h, Acc |
| Add constant to memory | No equivalent | ADDM #0FFh, dmad |
| Increment ARx | MAR *,ARx+ | MAR *ARx+ |
| Decrement ARx | MAR *, ARx- | MAR *ARx- |
| Negate accumulator | NEG | NEG Acc |

Here x is a number between 0 and 7 and ARx is one of the current auxiliary registers AR0 to AR7. dmad = data memory address and Acc = accumulator A or B.

Because the TMS320C5000 has two accumulators, it takes and extra operand compared to the single one used with earlier devices.

The usage of the instruction MAR (modify auxiliary register) has changed between the earlier devices and the TMS320C5000. On the earlier devices, the instruction was mostly used to change the current auxiliary register. However, on the TMS320C5000 it is mostly used to alter the value of an auxiliary register.

Unlike earlier devices, the TMS320C5000 can now directly add a positive or negative constant to the contents of a data memory address using the instruction ADDM (add long constant to memory). To carry out the same operation using an earlier processor was considerably less efficient because arithmetic operations could only be carried out in the accumulator. This meant copying the contents of the data memory address to the accumulator, performing the operation and then putting the result back in the data memory address.

Questions

| 1. | How do we add the constant 7Fh to accumulator A? |
| --- | --- |
| 2. | What value will be in accumulator A after the following sequence of instructions has been executed? |
|  | RSBX SXM LD #1, A ADD #0FFFFh, A |
| 3. | Which of the following does the instruction SUB #100h, A perform? a) constant 100h - accumulator A b) accumulator A - constant 100h |
| 4. | What value will accumulator B contain after the following instructions have been executed? RSBX SXM LD #0FFFFh, B SUB #1, B |
| 5. | The instruction ADDM #10, *AR4 means: a) Add long immediate value 10 to accumulator A b) Add long immediate value 10 to memory address pointed to by AR4 c) Add long immediate value 10 to memory-mapped register AR4? |
| 6. | What is the difference in meaning in the letter M in the instructions STM and ADDM? |
| 7. | How do we add 1 to the contents of data memory address 400h without using an accumulator? |
| 8. | There is no instruction to subtract a value from the contents of a data memory address. How do we subtract 1 from the value contained at data memory address 300h without using an accumulator? |
| 9. | The instruction MAR means: |
|  | a) move auxiliary register b) modify auxiliary register c) multiply auxiliary register? |
| 10. | How does the usage of the symbol * differ between the following: |
|  | a) LD *AR2+ b) MAR *AR2+ |
| 11. | What is the difference between the instruction MAR and the instruction STM? |
| 12. | What is the difference between the instructions CMPL and NEG? |