# Answers to Tutorial 8

Tutorial 8	Multiplications

| 1. | The temporary register is often abbreviated to the T register. |
| --- | --- |
| 2. | a) STM #45h, T ; Correct. b) LD #22h, T ; Should be STM #22h, T. c) LD 5h, T ; Correct. Direct addressing. d) LD *AR4, T ; Correct. Indirect addressing. e) LD *AR1+, T ; Incorrect. Modification not allowed. |
| 3. | To clear the T register to zero we can write: STM #0, T |
| 4. | The instruction MPY is used to multiply two signed numbers together. The largest positive number that can be multiplied is 7FFFh. The instruction MPYU is used to multiply two unsigned numbers when either of the values is in excess of 7FFFh. |
| 5. | a) MPYU #100, A ; Immediate addressing not supported. b) MPYU *AR3, B ; Correct c) MPYU 70h, A ; Correct d) MPYU *AR6-, A ; Correct |
| 6. | To multiply two 16-bit variables together we must first load one of them into the T register e.g.: LD *AR2, T ; Load first variable into T register. MPY *AR3,B ; Multiply second variable by T register. |
| 7. | When multiplying a series of values, it is more code-efficient to use indirect addressing to increment the pointer to the value in data memory e.g.: MPY *AR2+, #100h, A ; Point to next dmad. MPY *AR2, #200h, B. |
| 8. | The following two instructions: LT 0h, T MPY #10h, B can be reduced to: MPY 0h, #10h, B |
| 9. | The instruction MPYA (multiply by accumulator A) can only be used with accumulator A and therefore cannot be used with accumulator B. |
| 10. | The instruction MPYA multiplies the high word of accumulator A by the T register. When loading a value into accumulator A, a shift of 16 to the left must be used e.g.: LD *AR2, T ; Load the T register. LD #300h, 16, A ; Load high word of accumulator A. MPYA B ; Multiply high word of accumulator ; A by the T register. Put the ; product in accumulator B. |