# Answers to Tutorial 9

Tutorial 9.	Multiplications with Accumulation

| 1. | Accumulation means that the result of an operation is added to the on-going result of the previous operations. |
| --- | --- |
| 2. | When using indirect addressing, as part of an instruction we can increment or decrement the auxiliary register used as the pointer to a data memory address. We can also put the data handling instructions into a control loop. To do the same using direct addressing, each individual instruction would have to be written out in full. |
| 3. | Overflow means that the result of an operation has exceeded either the upper or lower limit allowed by that operation. |
| 4. | Accumulation can cause an accumulator to overflow. When overflow occurs in an accumulator, the contents of the accumulator become unreliable. A positive number can overflow to a negative number and a negative number can overflow in the opposite direction to generate a positive number. |
| 5. | To prevent overflow during accumulation we turn on overflow mode. |
| 6. | The instruction MAC is more code-efficient than the instructions MPY and ADD. Furthermore it can be used with a repeat instruction such as RPT, where the MPY and ADD must use BANZ. |
| 7. | The term saturation when applied to multiplication means that the product is limited to an upper thresholds of 7FFFFFFFh and a lower threshold of 80000000h. This prevents a multiplication overflowing to turn a positive number into a negative number or a overflowing in the opposite direction to turn a negative number into a positive number. |
| 8. | To turn on overflow mode we use the instruction SSBX OVM. |
| 9. | To turn off overflow mode we use the instruction RSBX OVM. |
| 10. | The following two instructions MPY *AR3+,A ADD A,B can be condensed into MAC *AR3+, B |
| 11. | The following three instructions: LD *AR3, T MPY *AR4, A ADD A, B can be condensed into: MAC *AR3, *AR4, B |
| 11. | The instruction MACA (Multiply by accumulator A and accumulate) uses the high word of accumulator A. Correct usage is as follows: RSBX SXM LD #200h, 16, A ; Load accumulator A with shift. MACA 80h, B ; Multiply and accumulate. |