# Laboratories 1 – 4 for the TMS320C5416 DSK

The first four in a series of laboratories to allow the student to answer some questions about sampling and FIR filters.

| 1. | What sampling rate in kHz is required for acceptable sound quality: a) speech b) music? |
| --- | --- |
| 2. | How many bits of resolution are required for acceptable sound quality: - speech - music? |
| 3. | What is the difference in sound between Rectangular, Hamming, Hanning, Blackman and Kaiser windows used to implement Finite Impulse Response (FIR) filters? |
| 4. | How many coefficients are required when implementing a FIR filter for clear division between one range and another? |

Each of the four questions can be answered by carrying out a different laboratory experiment. The answers are important for the design of audio systems.

Feedback to author: Richard@sikora.demon.co.uk

## Laboratory 1

| Objectives: | To determine the sampling rate in kHz required for acceptable quality transmission of the following: - speech - music |
| --- | --- |
| Equipment Required: | TMS320C5416 DSK CD Player or walkman Connecting cable 1/8 inch jack (3.5mm) to 1/8 inch stereo jack (3.5mm) Headphones or passive computer speakers or powered computer speakers Microphone |
| Code Source: | ### sampling_rate.zip |
| Installing the Application: | ### Install the software from the file sampling_rate.zip using the procedure described in the document Installing the TMS320C5416 DSK Applications |
| Experiment: | ### Connect the CD player / walkman to the LINE IN jack connector of the DSK using the connecting cable. Connect the loudspeakers. Set the output level of the CD player / walkman so that 3 LEDs are on. Run the TMS320C5416 DSK using speech (both male and female) from the CD player / walkman / microphone. Alter the settings of the 4 user switches on the DSK to change the sampling rate between 2 kHz and 48 kHz. Run the TMS320C5416 DSK using music with a range of different musical instruments from the CD player / walkman. Alter the settings of the 4 user switches on the DSK to change the sampling rate between 2 kHz and 48 kHz. |
| Questions: | ### What settings give acceptable speech for say a telephone answering machine? ### What settings are required for a clearly intelligible telephone conversation? What sampling rate is the minimum acceptable for music playback? |
| Reference: | ### A Simple Approach to Digital Signal Processing by Craig Marven and Gillian Ewers. Chapter 3. Converting Analog to Digital. |

## Laboratory 2

| Objectives: | To determine the number of bits of resolution required for acceptable quality transmission of the following: a) speech b) music |
| --- | --- |
| Equipment Required: | TMS320C5416 DSK CD Player or walkman Connecting cable 1/8 inch jack (3.5mm) to 1/8 inch stereo jack (3.5mm) Headphones or powered computer speakers or passive computer speakers Microphone |
| Code Source: | ### sampling_bits.zip |
| Installing the Application: | ### Install the software from the file sampling_bits.zip using the procedure described in the document Installing the TMS320C5416 DSK Applications |
| Experiment: | ### Connect the CD player / walkman to the DSK with the connecting cable. Connect the loudspeakers. Set the user switches on the DSK to position zero and adjust the output of the CD player / walkman so that 3 LEDs are on. Run the TMS320C5416 DSK using speech (both male and female) from the CD player / walkman / microphone. Alter the settings of the 4 user switches on the DSK to change the sampling size between 5-bits and 16-bits. Run the TMS320C5416 DSK using music with a range of different musical instruments from the CD player / walkman. Alter the settings of the 4 user switches on the DSK to change the number of sampling bits between 5 bits and 16 bits. Reduce the input signal level so the LEDs no longer come on. Alter the number of sampling bits between 5 bits and 16 bits. |
| Questions: | ### How many bits give acceptable speech for say a telephone answering machine? ### What settings give acceptable quality for a telephone conversation? What is the minimum number of bits required for acceptable music playback? How many bits are required to give good quality music playback? How does the number of bits affect the background noise, especially when the input signal is small? |
| Reference: | ### A Simple Approach to Digital Signal Processing by Craig Marven and Gillian Ewers. Chapter 3. Converting Analog to Digital. |

## Laboratory 3

| Objectives: | To hear the difference is sound between Finite Impulse Response (FIR) high pass and low pass filters implemented using the following windows: - Rectangular - Hamming - Hanning - Blackman - Kaiser |
| --- | --- |
| Equipment Required: | TMS320C5416 DSK CD Player or walkman Connecting cable 1/8 inch jack (3.5mm) to 1/8 inch stereo jack (3.5mm) Headphones or passive computer speakers or powered computer speakers Microphone Optionally, a waveform generator capable of generating sine waves between 20 Hz and 50 kHz. |
| Code Source: | ### FIR_windows.zip |
| Installing the Application: | ### Install the software from the file FIR_windows.zip using the procedure described in the document Installing the TMS320C5416 DSK Applications. This contains a .asm file. |
| Experiment: | ### Connect the CD player / walkman / microphone to the DSK with the connecting cable. Connect the speakers. Set the user switches on the DSK to position zero and adjust the output of the CD player / walkman so that 3 LEDs are on. Run the TMS320C5416 DSK using speech (both male and female) from the CD player / walkman. Alter the settings of the 4 user switches on the DSK to change type of FIR filter. Ideally, a low pass filter should allow bass through only, but no treble. Similarly, a high pass should allow treble through only, but no bass. There should also be minimum distortion. Run the TMS320C5416 DSK using music with a range of different instruments from the CD player / walkman. Alter the settings of the 4 user switches on the DSK to change the type of FIR window. If a waveform generator is available, plot the frequency response for each type of window. The bargraph on the DSK is calibrated in 6 dB intervals. |
| Questions: | ### Which FIR window types are best for implementing low pass and high pass filters for speech? ### Which FIR window function is the best for music playback? |
| Associated Document: | ### FIR Windows.doc |
| Reference: | ### Digital Signal Processing, A Practical Approach, Second Edition by Emmanuel C. Ifeachor and Barrie W. Jervis. Chapter 6. A framework for digital filter design. Chapter 7. Finite impulse response (FIR) filter design. |

## Laboratory 4

| Objectives: | To determine the number of coefficients required to implement high pass and low pass Finite Impulse Response (FIR) filters with acceptable audio quality. |
| --- | --- |
| Equipment Required: | TMS320C5416 DSK CD Player or walkman Connecting cable 1/8 inch jack (3.5mm) to 1/8 inch stereo jack (3.5mm) Headphones or passive computer speakers or powered computer speakers Microphone Optionally, a signal generator capable of generating sine waves between 20 Hz and 50 kHz and a connecting cable between the signal generator and the DSK. |
| Code Source: | ### FIR_length.zip |
| Installing the Application: | ### Install the software from the file FIR_length.zip using the procedure described in the document Installing the TMS320C5416 DSK Applications. There are .asm files |
| Experiment: | Connect the CD player / walkman to the DSK with the connecting cable. Connect the speakers. Set the user switches on the DSK to position zero and adjust the output of the CD player / walkman so that 3 LEDs are on. Run the TMS320C5416 DSK using speech (both male and female) from the CD player / walkman / microphone. Alter the settings of the 4 user switches on the DSK to change the number of coefficients used to implement the FIR filter. Ideally, a low pass filter should allow bass through only, but no treble. Similarly, a high pass should allow treble through only, but no bass. There should also be minimum distortion. If treble can be clearly heard on the low pass filter, then there are insufficient FIR coefficients. In a similar way, bass coming though a treble speaker also means insufficient FIR coefficients. Run the TMS320C5416 DSK using music with a range of different instruments from the CD player / walkman. Alter the settings of the 4 user switches on the DSK to change the type of FIR window. Make a note of the number of coefficients required for satisfactory performance. As part of the experiment, the number of coefficients can be fine tuned to find out more precisely how many coefficients are required. For example, the number 41 in the code in the function FIR_dual_filter_variable_asm in FIR_length.c can be changed to a value in the range 3 to 51. dual_output = FIR_dual_filter_variable_asm ( &blackman_low_pass_filter_600Hz[0], mono_input, 25); /* Uses 25 coefficients */ If a waveform generator is available, plot the frequency response curves for various filter window lengths. |
| Further Experiments: | ### If a signal generator is available, plot the frequency response of the different filter lengths. The bargraph is calibrated in 6 dB intervals. |
| Questions: | ### How many coefficients are required for clear division between bass and treble? What happens to the transition between bass and treble when there are not enough coefficients? |
| Associated Document: | ### FIR Length.doc |
| Reference: | ### Digital Signal Processing, A Practical Approach, Second Edition by Emmanuel C. Ifeachor and Barrie W. Jervis. Chapter 6. A framework for digital filter design Chapter 7. Finite impulse response (FIR) filter design. |