| Pin | Name | Description |
| --- | --- | --- |
| 1 | RESET | Reset Signal, Initialises the LCD when Low (Active Low) |
| 2 | GND | Ground |
| 3 | VCC | Supply Voltage 2.2 Volts to 3.6 Volts, nominal 3.3 Volts |
| 4 | CS | Chip Select Signal (Active Low) |
| 5 | RS | Register/Control Select. Low = Register Index, Hi = Data |
| 6 | WR | Write Signal Strobe (Active low) |
| 7 | RD | Read Signal Strobe (Active Low) |
| 8-15 | D0-D7 | LCD DATA Bus, connects to host micro 8 bit data bus or port |
| 16 | GND | Ground |
| 17 | NC | Not Used |
| 18 | LED+ | Backlight LED positive supply 10mA to 70mA |
| 19 | LED- | Backlight LED negative supply, nominally connects to GND |

Product Overview:

The LCD-128 is a 1.5” 128x128 CSTN colour LCD display. This LCD is the same display used in the microLCD range of modules. It allows a very cost effective means of adding a full colour small display to any product or your next microcontroller project. Schematics and sample initialisation code are also available upon purchase.

Main Features:

- 128 x 128 1.5” CSTN 65K colours

- HD667A66 Driver IC (Data Sheet available)

- 19 pins on LCD FPC, 0.5mm pitch

- Dimensions: 38.6mm x 37.6mm x 4mm

- 2.2 to 3.6 Volts supply, nominal 3.3 Volts @ 1mA

- 5 Volts or greater for the White LED backlighting @ 10mA to 70mA

4D Systems Pty. Ltd.           support@4dsystems.com.au             www.4dsystems.com.au

For Help and Other Information:

- Assistance with latest information and downloads visit the μLCD product web-page of your distributor.

- Questions and technical support please email support@4dsystems.com.au

Related Products:

- uLCD-128-BREAKOUT (Carrier Board for the 128x128 LCD display)

- LCD-160 (1.8” 128x160 RGB 65K colour LCD Display)

- uLCD-160-BREAKOUT (Carrier Board for the 128x160 LCD display)